How to change your router admin password safely

https://cyberguardlab.com/blog/router-admin-password-change

Independent US household comparison site. Not the vendor. Paid links below may earn a commission. Confirm first-year price, renewal, device limit, and refund terms at checkout.

Change your router admin password (the login that opens the router’s settings web UI or app)—not only the Wi-Fi passphrase guests type. Use a unique, long password stored in a password manager, and finish by updating firmware. This how-to is router administration credentials—not turning on WPA3 / Wi-Fi encryption, not Private DNS on Android, and not ISP bill phishing.

## 1. Why the admin password is different from Wi-Fi

Wi-Fi passwords let devices join the network. The admin password lets someone change DNS, open ports, install malicious firmware settings, or create a hidden guest network. Default stickers like `admin/admin` are still common on older gear.

Paid link

Norton 360  |  See the Norton first-year offer

See the Norton first-year offer

Offer page: https://cyberguardlab.com/norton

Callers who demand the admin password to “fix your slow Wi-Fi” are a common social-engineering path. Hang up; use the number on your bill or ISP app.

## 2. Safe vs unsafe admin habits

Habit | Safer | Risky

Where you log in | LAN IP you typed / official app | “Router support” link from cold email

Password | Unique long admin secret | Same as Wi-Fi or printed default forever

Remote WAN admin | Off unless required + vendor MFA | Open to the internet with default creds

Firmware | Vendor-signed updates | Third-party “optimizer” firmware files from forums

## 3. Steps: change the admin password safely

Prefer Ethernet from a PC to the router for this session when possible.

Find the LAN admin URL from the sticker or manual (often `192.168.0.1`, `192.168.1.1`, or a vendor `.local` name)—type it; do not use a random SMS link.

Sign in with the current admin credentials. If unknown, use the vendor’s documented reset procedure (this wipes settings).

Immediately set a new admin password (and admin username if the firmware allows). Use a password manager.

Turn off remote administration / WAN-side management if you do not need it.

Change the Wi-Fi passphrase separately if it was ever shared widely—see also WPA3.

Check for firmware updates in the same admin UI or the vendor’s official app/site.

Save a configuration backup only to encrypted storage you control; note the new password offline.

Reboot and confirm you can still log in; reconnect devices.

## 4. After a suspected compromise

Factory-reset the router, reinstall settings from scratch, and set a new admin password.

Change Wi-Fi passphrases; revisit IoT update status.

Review DNS settings for unfamiliar resolvers.

Scan PCs with Norton or Bitdefender; keep one primary product (TotalAV, McAfee, Avast).

## FAQ

### I changed Wi-Fi—does that change admin too?

Usually no. Change admin separately inside the router UI.

### Is this the same as WPA3?

No. WPA3 is wireless encryption between clients and the access point. Admin password guards the settings panel.

### ISP gateway vs my own router?

If the ISP device is your only gateway, use their app/portal or documented admin page. Some ISP units lock admin; ask the ISP for the procedure.

### Can I email the password to myself?

Prefer a password manager. Email is a weak vault.

## If you still want a paid suite

Avoiding how to change your router admin password safely does not require paid antivirus. Careful habits and official support paths are a valid buy-nothing stack. If you want multi-device paid protection later, use only these on-site paid links: TotalAV (paid link), Norton (paid link), Bitdefender (paid link), McAfee (paid link), Avast (paid link).

## Sources

CISA: Securing home networks / routers (guidance hub)

FTC: Secure your router

Example vendor help (Netgear admin password)

Example vendor help (TP-Link login)

## Also read

How to turn on WPA3 or the strongest Wi-Fi encryption you have

How to update home router firmware

How to set up a guest Wi-Fi network at home

How to segment smart TVs and cameras on a separate VLAN

How to spot a fake ISP outage or bill phishing email